# Analytical Longitudinal Speed Planning for CAVs with Previewed Road Geometry and Friction Constraints

Liming Gao, Craig Beal, Daniel Fescenmyer, Sean Brennan, IEEE Member

.Abstract— Due to the lack of information, current vehicle control systems generally assume that the road friction conditions ahead of a vehicle are unchanged relative to those at the vehicle's position. However, with connectivity either to other vehicles, infrastructure, or cloud services, future vehicles may have access to this information which is particularly valuable for planning velocity trajectories ahead. This paper introduces a method for planning longitudinal speed profiles for Connected and Autonomous Vehicles (CAVs) that have previewed information about road geometry and friction conditions. The novelty of this approach is to explicitly include consideration of the friction ellipse available along the intended path to develop an analytical solution to the allowable vehicle velocity profile that prevents departure from the friction ellipse. The results further define the relationship between the vehicle speed, path geometry, and minimum friction preview distance that ensure the vehicle has sufficient time to take action for upcoming hazardous situations. The efficacy of the algorithm is demonstrated through an application case where a vehicle navigates a curving road with changing friction conditions at maximum allowable and stable speeds, with results showing that the vehicle consistently operates within the available friction limits.

# INTRODUCTION

Highway road conditions are reported to have a significant influence on traffic safety and mobility. Unforeseen hazardous road conditions such as snow, ice, rain, etc potentially can have a significant negative impact on vehicle safety and mobility. The Federal Highway Administration reported that approximately 20% of all crashes occur in adverse road conditions; light rain or snow can reduce average traffic flow volume by 5%-10% on the highway [1]. Conversely, efficient highway systems should ensure that vehicles travel as expediently as possible to maximize the throughput of the highway. Uncertainty about exact highway conditions creates a conflict between these two objectives, as drivers may drive too fast and experience adverse road conditions or drive more conservatively than necessary, impeding traffic flow.

Much research has been conducted to address the vehicle mobility issue in hazardous conditions. One common way is to develop Advanced Driver Assistance Systems (ADASs) such as EBS, LKA, and ESC, etc., which can assist a driver to follow the desired path stably in hazardous situations. These assistance control systems designed for the vehicle itself can be categorized into lateral control, longitudinal control, and hybrid control. Once the vehicle experiences instability, the controller can output a proper steering angle and/or driving/braking torque command to stabilize movement. For example, Yu, et al. [2] designed a feedback-feedforward steering controller to improve the vehicle stability when hard-braking maneuvers on road with split friction. Li, et al. presented a torque control strategy for the situation of abrupt changing of road friction [3].

However, most of these controllers are passively activated only when the vehicle states have a significant deviation from the nominal value. Even worse, a controller may fail to keep the vehicle stable if the road condition changes significantly and rapidly because of delays in the system and real actuator limitations. [ref]

In contrast with these control strategies that react to certain measured vehicle instabilities, researchers have also developed proactive envelope controllers which can be combined with real-time road friction estimation to keep the vehicle within a stable region bounded by road-tire friction limits. However, these systems only maintain vehicle stability from the perspective of body-fixed states and cannot prevent the vehicle from deviating from the desired path when the road condition changes significantly and rapidly.

Another method of proactively maintaining vehicle stability is to utilize longitudinal velocity planning to maintain stability when following a path with tight curvature change [4]. The idea is to reduce vehicle speed before a potentially dangerous situation is reached, in contrast with widely used stability control systems that only react once the loss of control is imminent. The reason why speed control is effective is that the vehicle has a larger stable operating region to follow the desired path with a smaller velocity [4][5].

However, previously published longitudinal velocity planning approaches consider only the variation of path curvature, and little attention has been paid to the road surface friction condition changing when conducting longitudinal velocity planning. Research has proved that the prior estimation of friction and peak tire force before the slick region is reached, allows a vehicle chassis control system to work more reliably and proactively [2]. As a result, if the vehicle can preview the path friction reduction and slow down appropriately before entering the slick region, even a common /simple driving algorithm can operate the vehicle to follow the desired path well.

Although each vehicle on a roadway can constantly measure and navigate the world using its in-vehicle systems, it is almost impossible for an individual vehicle to preview the road condition for its entire trip. One potential solution to preview the road surface friction condition is motivated by the increasing research into cyber-physical systems, especially Connected and Autonomous Vehicles (CAVs). This solution takes advantage of both network intelligence and individual intelligence, where road friction information [7][8] from the individual vehicles and roadside sensor units is shared within the network. Within the network, the information can flow between vehicle to vehicle, vehicle to infrastructure, and vehicle to database systems. In this way, each vehicle in the network can query the aggregated friction information from the shared database. An illustration of this network framework is shown in Fig. 1. It is anticipated that in the future, a vehicle enhanced by the network intelligence could move reliably even without an advanced ADAS.

- Figure 1.  A strategy of road friction and geometry preview with a shared roadway database. The shared database can leverage network intelligence to substantially improve the operation of each individual in the population.

The desired improvements in vehicle safety and mobility can be achieved by limiting the vehicle speed based on the previewed road condition. In this paper, an idea of road friction preview through database-informed CAVs is introduced. Then inspired by the work of various authors on vehicle dynamics at the limits of handing [9]–[11], we present an algorithm to generate a longitudinal vehicle velocity limit profile for a given desired path that takes into account local speed limits, road geometry, and current road friction conditions. Closed-form analytical formulae are established for certain common preview cases, allowing for the introduction of a straightforward and computationally efficient algorithm for generating the velocity profile. Furthermore, relationships are established between vehicle speed, path geometry, and minimum friction preview distance that ensure the vehicle has sufficient time to take action for upcoming hazardous situations.

The remainder of this paper is organized as follows: Section II discusses the velocity limit profile planning based on the road-tire friction limits and road geometry. Section III analyzes the friction preview distance. Section IV demonstrates the approach with an application case. Finally, Section V summarizes the main results of the work and anticipated future work.

# Vehicle Longitudinal Velocity Planning Given a Reference Path

This section presents the generation of a limit speed profile that vehicles can achieve without exceeding available tire friction limits constraints. The section begins by deriving the constraints of driving on the friction circle by controlling the vehicle longitudinal dynamics, and then introduces an approach to describe a driving path by station s, curvature κ, and friction coefficient μ. Finally, the section concludes with the details of the velocity planning method.

## A. Vehicle Chassis Model and Tire Friction Limits

The vehicle dynamic equations for the three states single track model shown in Fig. 2 are:

where longitudinal velocity Ux, lateral velocity Uy, and yaw rate r are the three states. θ is the path grade, and g is the gravitational acceleration. The vehicle parameters include the vehicle mass m, yaw moment of inertia Izz, front wheel steering angle δ, and a and b the distances from the vehicle’s center of gravity to the front and rear axle respectively. Fxf, Fxr, Fyf, Fyr are the longitudinal and lateral forces acting on the front and rear tires.

- Figure 2. Planar single-tack vehicle chasis model

The maximum available longitudinal force Fx and lateral force Fy at each tire is constrained by a circular friction constraint:

where μ is the road-tire friction coefficient, and Fzf and Fzr are the vertical force at the front and rear axle respectively.  Ignoring the effects of lateral and longitudinal load transfer, the vertical forces are, .

Determining the limit speed profile requires the vehicle to utilize all the available tire friction to generate forces so that vehicle can operate at the acceleration limits to achieve the maximum safe speed [12], [13]. It implies that all tire forces need to remain on the boundary of the friction circle, i.e. equality holds in .

Fig. 3 shows the road-tire force when maneuvering through a left corner of a path at the tire limits. The boundary of the friction circle depends on the road-tire friction coefficient.

- Figure 3. The maximum performance of vehicle can be achieved by driving at the boundary of friction circle. This plot shows the left cornering case.

As this work is interested in the longitudinal dynamics of a vehicle traveling stably along a path, the lateral and rotational state derivatives are assumed to be zero with quasi-steady values of the lateral and rotational velocities dictated by the road geometry:

where κ is the path curvature.

Substituting  with equality condition, and  into , , and  yields:

a first version of the key equation governing the longitudinal speed while operating at the friction limits on a path of known curvature. Before going further, an adjusting parameter λ is added into the equation . This parameter allows for compensation of the uncertainty of friction preview and the fact that most drivers may not be comfortable with or able to operate a vehicle at the friction limits as racing drivers or autonomous driving systems might [14]. Consequently, the dynamics equation that depicts the maximum available longitudinal acceleration is expressed as:

where the plus-minus sign (±) corresponds to acceleration and deceleration respectively. Positive θ corresponds to upgrades and negative is for downgrades. The parameters μ and κ in  depend on the path position, which is introduced in the following section.

## B. Path Representation

In contrast to many publications that describe the process of path planning for minimal time or maximal speed within the boundaries of a road or track, this paper instead focuses on following a given path. Specifically, the emphasized use is for following the lane centerline of a given highway with occasional vehicle-planned lane changes. Therefore, the desired path is assumed to be known. The clothoid path description is widely used for highway road design [15] and vehicle path planning, for example, the racing line [11], [16] and minimum curvature optimal path [17],[18]. A clothoid path can generally be described by a succession of turns - consisting of spirals and constant radius arcs - and straight lines. The spiral segments are defined to have linearly increasing or decreasing curvature along the path:

where s referred to as “station” in this paper, is the distance measured along a path; Ls is the total length of the spiral and κc is the curvature at the end of the spiral. In this way, the curvature of the whole path can be described by a succession of linear functions.

Without loss of generality, an example oval path similar to the Larson Institute Test Track [19] is shown in Fig. 4. The path is given according to the highway road design rule which decomposes the corner into three phases: an entry clothoid, a circular arc, and an exit clothoid [11]. The corresponding curvature, along with the road grade and a hypothetical friction profile, are shown in Fig. 5. All of these path parameters can be described as a function of the path station: κ(s), μ(s), θ(s). The portions of the path where the grade is non-zero was chosen to be similar to the off-ramp and on-ramp part of a highway

## C.	Velocity Profile Generation

Velocity planning has a significant impact on driving safety, especially when vehicles drive on a road with changing friction and geometry. With the longitudinal dynamic equation  and the desired path description, the speed profile can be determined. The approach presented in this paper is inspired by previous work that addressed the profile determination via three passes [13], nonlinear optimization [4], and segment and iteration [20], where a velocity profile is planned given the path curvature.

- Figure 4. A circular oval smaple path.

- Figure 5. The curvature, previewed friction coefficient, and previewed grade for the sample path. The friction is assumed to change abruptly. (10% is too large, 6% is maximum)

The first step of the approach is to express the longitudinal acceleration with respect to the vehicle station:

Substituting  into  yields:

For the radius arc path whose curvature is a constant, the stop distance can be determined by solving  analytically. for the spiral case, a polynomial approximation solution can be found. (may update following numerical method to piecewise analytical solution method,although it is a little hard) For general case,  can be solved using a numerical integration method.

where . The solution will be accurate enough if path waypoints are dense, i.e., the is small enough. In this paper, we choose  smaller than 0.1m. λ=0.95 is also chosen to explore the vehicle behavior close to the friction limits.

The first step of generating the speed profile is to find the maximum permissible steady-state vehicle velocity with zero longitudinal acceleration, which is given by :

Notice that the steady-state speed will be very high when the curvature is small (the curvature is zero, yielding an infinite maximum curve speed for a straight line) and thus a speed limit of 55 m/s was imposed. The first step result is shown as the “curve limit speed” in Fig. 6(a).

The following next step is a forward integral step:

It starts from the vehicle's current speed. At each step, the result is compared to the curve limit speed, and the minimum value is taken. This step indicates how fast a vehicle can accelerate.

The final step is a backward integral step:

It starts from the maximum allowable vehicle speed at the end of the path and back toward to current station. At each step, the result is compared to the forward integral results. This step indicates how fast a vehicle can decelerate.

In summary, the feasible speed trajectories and associated preview are determined as follows:

- Break the path ahead of the vehicle into discrete locations in position. In this work, (delta s) = 0.1m.

- The road friction, mu, and road curvature, kappa, values are found far ahead of the vehicle, much farther than the possible minimum look-ahead distance. These values are assumed to be stored either on the vehicle or obtained in advance via V2x communication. In the examples that follow, 2 km is used.

- The curve limit speed，???,  is found for every path location, s, as calculated by Equation (8). If the speed is above the speed limit, the speed limit is used (Equation????)

- Starting at s=0 and stepping in the forward direction, the possible change in speed is calculated from Equation (13) such that the friction ellipse is not violated in the forward-direction.

- Next, starting at s = s_final and stepping in the reverse direction, the change in speed is calculated from Equation (14) such that the friction ellipse is not violated in the backward direction.

- The minimum of speeds from step 4 and 5 are used as the final limit speed.

- Optionally, one can calculate next the acceleration profile in the x- and y-directions, as well as the total acceleration, a_total, for comparison to the possible acceleration available, a_max， whichis calculated from from road friction and grade in Equation (???)

- Figure 6. (a) The intermediate and final results of the speed profile of the computation algorithm. (b) The acceleration of the speed profile.

- =

The speed profile results and the acceleration for the example path are shown in Fig. 6. Fig. 6 (b) indicates that the total acceleration  is bounded by the friction limit and road grade μgcos(θ)+g|sin(θ)|. Also, note that the acceleration value is as large as a racing car especially at the large friction region, but the acceleration value can be tailored by choosing a proper adjusting parameter λ.

# Calculation of Minimum Friction Preview Distance

Examining fig.6, one can see that the spped profile calculated at each station requires information ahead of the station, but the distance ahead changes at each station depending on conditions, to illustrate this, fig.XX shows the distance ahead of each station that must be known in order for the velocity limits at the station to be correctly calculated. We call this information window the minimum preview distance.

(put figure here showing preview distance versus station)

An important aspect of the preview distance is that it changes considerably versus the road friction values and the road curvature. For example, for curve-keeping, a straight road would not require any preview. The dependence of preview on friction is also shown in (figure XXX) where different friction assumptions yield different preview curves.

When friction information is collected via a system outside the vehicle, for example a shared database, the use of an appropriate preview distance is important to prevent unnecessary transmission of large amounts of data. Longer preview distances involve more data transmission, resulting in more time delay and data cost, but an overly short preview distance could lead to a situation where a vehicle might not have enough space to respond to the dangerous situation ahead. However, the database or the vehicle can host algorithms that calculate the minimum preview horizon as noted earlier. This section presents methods for determining the minimum friction preview distance for different situations: maximum velocity preview distances (e.g., the results of the prior sections), or even the preview distances needed to achieve braking to a complete stop. An interesting result of this work is the finding that many roads are designed such that there is a minimum friction value for operation such that, if actual friction values are above this value, then no preview is needed for maintaining the speed limits.

## A.	Minimum Friction Preview Distance

The criterion for determining the preview distance in this paper is that a vehicle could have sufficient to make a decision for deceleration. A confident lower friction bound which is less than actual friction can be assumed reasonably before driving, and thereby a lower bouned speed profile can be calculated using the method in the previous section. For example, there is no icy road (friction coefficient is 0.1) during summer in PA, US. An example of a speed profile with lower friction bound of  is shown in figure 7. If a vehicle moves with a speed slower than the lower speed bound, then no further friction preview query from the shared database is required,i.e. friction preview distance dp is zero. With this assumption, our task is to find the vehicle deceleration distance from current speed to confident speed bound.

- Figure 7. An arc path with Rc = 200m and assumed lower bounded friction λμ=0.2.

For the radius arc path, the deceleration distance can be determined by solving  analytically:

where Ux_a0 is the initial speed and Ux_af is the final speed. Rc=1/κc is arc radius.

For conservation of friction preview distance calculation, the friction μ used is assumed to be . The final speed is the confident speed bound Then, we have the estimation of minimum friction preview distance:

Equation (16) indicates that the La depends on Ux_a0, and Rc.

- Figure 8. The relation between friction preview distance, current velocity and confident friction bound for a path with Rc = 200m.

Fig. 8 indicates that the friction preview distance increases with vehicle current speed but decreases with confident friction bound.

For comparison, most human drivers with the required driving vision can only see the road approximately 500m ahead, even on a clear day [ref]. This visibility is also reduced significantly during darkness and adverse weather, such as fog, rain, and snow, where hazardous road friction conditions often occur. The driver’s sight is further limited by road geometry such as corners and crests of hills. Thus, the preview of road friction and geometry could provide significant benefit for drivers to adjust their driving speed proactively.

The criterion for determining the preview distance in this paper is that a vehicle could always stop within the distance. The strategy is to calculate the maximum stop distance with the most critical scenario for conservation. Therefore, we assume that the vehicle is driving on a snowy road with a constant allowed road-tire friction coefficient value, i.e. the product λμ=0.2, and with a maximum permissible initial speed. With this assumption, our task is to find the scenario where the vehicle decelerates to stop with the longest distance.

Section II.B introduced the convention that a path comprises straight lines with zero curvature, radius arcs with constant curvature, and spirals with linear curvature. Thus, for a given path, the straight-line segment allows the highest permissible initial speed. According to , the portion of a path with larger curvature has a smaller maximum permissible initial speed. And for a given initial speed, a larger curvature path results in a smaller deceleration and thereby a longer stop distance. Applying this analysis, the most critical scenario is a vehicle driving at the speed limit Ux0 on a straight-line road, entering a spiral road, and finally stopping at the arc segment, which is similar to a highway off-ramp road and is shown as Fig. 7.

- Figure 7. The scenario where a vehicle decelerates starts from a line segment with an initial speed Ux0, then enters a piral segment with length ds, and finally stops at the arc segment with radius Rc=1/κc. (this description is not clear. Can say: the most critical scenario is that a vehicle decelerates with an initial speed Ux0, finally stop at the red point)

For the straight line segment, the deceleration distance can be determined by solving  analytically:

where Ux_lf is the final velocity of the line segment and the initial velocity of the spiral segment, which can be determined as follows. For the spiral path, we suppose its length isand derive the numerical solution of Ux_lf from :

for k = 0, 1 ,2 , 3 , … , N= Ls/Δs, where , and s0=0, Ux(sN) = Ux_lf , and Ux(s0) = Ux_a0 is the initial speed of arc path segment, which is determined as follows.

For the radius arc path whose curvature is a constant, the stopping distance can be determined by solving  analytically:

(full solution)

Equation  implies that the stopping distance is increasing with the increase of Ux0, but La has a supremum when :

The supremum is independent of friction μ, and the reason is that the initial speed on the circular arc is limited by curvature and friction. This supremum means that we can calculate the stopping distance by accelerating the vehicle from the critical stop point shown in Fig. 7 until Ux0 is reached. The acceleration distance is the addition of La, Ls, Ll:

And the acceleration distance is the longest stopping distance and thereby minimum preview distance for certain Ux0, κc, and Ls.

With a range of reasonable values of Rc=1/κc and Ls following the highway design rules[15], for a fixed value of Ux0=40m/s, we can determine the relationship between the total acceleration distance i.e. minimum preview distance  and Rc, Ls, which is shown in Fig. 8.

Fig. 8 indicates that the longest stopping distance is achieved when both Rc, Ls are relatively large. Examining the numerical values, it can be seen that the maximum stopping distance is less than 750m if the Ls is less than 400m and Rc is less than 400 m. This conclusion works for all initial speeds less than the speed limit of 40m/s.

On the other hand, the preview distance can also be calculated with different Ux0  but fixed Rc and Ls using  For the sample path shown in Fig.4, whose Rc_max= 200m and Ls_max= 145m, the result is shown in Fig. 9.

In conclusion, the preview distance can be calculated using  given vehicle initial speed Ux0, path parameters: Rc_max, and Ls_max, assumed minimum friction coefficient μmin, and a conservative constant c.

- Figure 8. The stopping distance on entering clothoid path with an initial speed Ux0 = 40m/s. In this plot, Rc = 1/κc.

- Figure 9. The minimum preview distance with different initial speeds for the sample path (Rc_max= 200m and Ls_max= 145m) shown in Fig.4

For comparison, most human drivers with the required driving vision can only see the road approximately 500m ahead, even on a clear day [ref]. This visibility is also reduced significantly during darkness and adverse weather, such as fog, rain, and snow, where hazardous road friction conditions often occur. The driver’s sight is further limited by road geometry such as corners and crests of hills. Thus, the preview of road friction and geometry could provide significant benefit for drivers to adjust their driving speed proactively.

# Application Case and Simulation Results

This work presents an application case to demonstrate the efficacy of velocity profile planning. In this case, a vehicle follows the path shown in Fig. 4 with the hypothetical station-dependent friction conditions given in Fig. 5.

In open-loop driving with the vehicle on a low-friction curve, drivers are often unable to stabilize the vehicle. But with the preview of path friction and geometry, a speed controller can plan the velocity profile to prevent the vehicle from excess speed in a curve and thereby preventing poor trajectory tracking or even road departure.

A driver model is used based on the assumption that a driver intends to look ahead with a 20m preview horizon when tracking the desired path. The controller can follow the path well with the preview of friction. Use adjusting parameter λ=0.5 for a non-aggressive driver style.

- Figure 10. (this plot is just an example, a correct one will be added) The lateral tracking error for the situation with and without friction preview is evident, showing that the vehicle can track the desired trajectory with much less lateral error with friction preview.

# Conclusion

This paper has introduced a method for planning longitudinal speed profiles for CAVs that have previewed information about road geometry and friction conditions. The idea of preview is to extend individual intelligence with network intelligence.  The longitudinal speed planning is enabled by solutions to the allowable velocity profile that prevents departure from the friction ellipse with closed-form analytical results in certain cases. The results further define the relationship between the minimum preview distance and longitudinal velocity limits that ensure the vehicle has sufficient time to take action for upcoming hazardous situations. The efficacy of the algorithm is demonstrated through an application case where a vehicle navigates curvy roads with changing friction conditions at maximum speeds.

In analyzing the preview distance, this paper presents the most conservative or circular results. Future work will include a more precise and less conservative preview distance, taking into account a variety of driving conditions.

Acknowledgment

This material is based upon work supported by the National Science Foundation under grant no. CNS-1932509 CNS-1931927, CNS-1932138 “CPS: Medium: Collaborative Research: Automated Discovery of Data Validity for Safety-Critical Feedback Control in a Population of Connected Vehicles”.

References

[1]	A. F. Highway, “How Do Weather Events Impact Roads?,” 2020. https://ops.fhwa.dot.gov/weather/q1_roadimpact.htm.

[2]	L. Yu, S. Zheng, Y. Dai, L. Abi, X. Liu, and S. Cheng, “A feedback-feedforward steering controller designed for vehicle lane keeping in hard-braking manoeuvres on split-μ roads,” Veh. Syst. Dyn., 2021, doi: 10.1080/00423114.2020.1869274.

[3]	L. Li, “PID PLUS FUZZY LOGIC METHOD FOR TORQUE CONTROL IN TRACTION CONTROL SYSTEM,” Int. J. Automot. Technol., vol. 13, no. 3, pp. 441–450, 2012, doi: 10.1007/s12239.

[4]	H. Cao, X. Song, S. Zhao, S. Bao, and Z. Huang, “An optimal model-based trajectory following architecture synthesising the lateral adaptive preview strategy and longitudinal velocity planning for highly automated vehicle,” Veh. Syst. Dyn., vol. 55, no. 8, pp. 1143–1188, 2017, doi: 10.1080/00423114.2017.1305114.

[5]	C. E. Beal and C. Boyd, “Coupled lateral-longitudinal vehicle dynamics and control design with three-dimensional state portraits,” Veh. Syst. Dyn., vol. 57, no. 2, pp. 286–313, 2019, doi: 10.1080/00423114.2018.1467019.

[6]	C. G. Bobier-Tiu, C. E. Beal, J. C. Kegelman, R. Y. Hindiyeh, and J. C. Gerdes, “Vehicle control synthesis using phase portraits of planar dynamics,” Veh. Syst. Dyn., vol. 57, no. 9, pp. 1318–1337, 2019, doi: 10.1080/00423114.2018.1502456.

[7]	S. Roychowdhury, M. Zhao, A. Wallin, N. Ohlsson, and M. Jonasson, “Machine Learning Models for Road Surface and Friction Estimation using Front-Camera Images,” in 2018 International Joint Conference on Neural Networks (IJCNN), 2018, pp. 1–8, doi: 10.1109/IJCNN.2018.8489188.

[8]	C. E. Beal, “Rapid Road Friction Estimation using Independent Left/Right Steering Torque Measurements,” Veh. Syst. Dyn., vol. 58, no. 3, pp. 377–403, 2020, doi: 10.1080/00423114.2019.1580377.

[9]	J. Subosits and J. C. Gerdes, “Autonomous vehicle control for emergency maneuvers: The effect of topography,” Proc. Am. Control Conf., vol. 2015-July, pp. 1405–1410, 2015, doi: 10.1109/ACC.2015.7170930.

[10]	K. Kritayakirana and J. C. Gerdes, “Autonomous vehicle control at the limits of handling,” Int. J. Veh. Auton. Syst., vol. 10, no. 4, pp. 271–296, 2012, doi: 10.1504/IJVAS.2012.051270.

[11]	P. A. Theodosis and J. C. Gerdes, “Nonlinear optimization of a racing line for an autonomous racecar using professional driving techniques,” in ASME 2012 5th Annual Dynamic Systems and Control Conference Joint with the JSME 2012 11th Motion and Vibration Conference, DSCC 2012-MOVIC 2012, 2012, vol. 1, pp. 235–241, doi: 10.1115/DSCC2012-MOVIC2012-8620.

[12]	W. C. Mitchell, R. Schroer, and D. B. Grisez, “Driving the traction circle,” SAE Tech. Pap., no. 724, 2004, doi: 10.4271/2004-01-3545.

[13]	N. R. Kapania, J. Subosits, and J. C. Gerdes, “A Sequential Two-Step Algorithm for Fast Generation of Vehicle Racing Trajectories,” J. Dyn. Syst. Meas. Control. Trans. ASME, vol. 138, no. 9, 2016, doi: 10.1115/1.4033311.

[14]	C. E. Beal and J. C. Gerdes, “Model predictive control for vehicle stabilization at the limits of handling,” IEEE Trans. Control Syst. Technol., vol. 21, no. 4, pp. 1258–1269, 2013, doi: 10.1109/TCST.2012.2200826.

[15]	AASHTO, A Policy on Geometric Design of Highways and Streets. 2018.

[16]	P. A. Theodosis and J. C. Gerdes, “Generating a racing line for an autonomous racecar using professional driving techniques,” ASME 2011 Dyn. Syst. Control Conf. Bath/ASME Symp. Fluid Power Motion Control. DSCC 2011, vol. 2, pp. 853–860, 2011, doi: 10.1115/DSCC2011-6097.

[17]	J. Villagra, V. Milanés, J. Pérez, and J. Godoy, “Smooth path and speed planning for an automated public transport vehicle,” Rob. Auton. Syst., vol. 60, pp. 252–265, 2012, doi: 10.1016/j.robot.2011.11.001.

[18]	H. A. Hamersma and P. S. Els, “Longitudinal vehicle dynamics control for improved vehicle safety,” J. Terramechanics, vol. 54, pp. 19–36, 2014, doi: 10.1016/j.jterra.2014.04.002.

[19]	“Test Track.” https://www.larson.psu.edu/about/test-track.aspx (accessed Apr. 08, 2021).

[20]	R. Solea and U. Nunes, “Trajectory planning with velocity planner for fully-automated passenger vehicles,” IEEE Conf. Intell. Transp. Syst. Proceedings, ITSC, pp. 474–480, 2006, doi: 10.1109/itsc.2006.1706786.

Liming Gao is a graduate student in Mechanical Engineering at The Pennsylvania State University, University Park, PA 16802, USA (e-mail: lug358@ psu.edu).

Craig Beal is with Department of Mechanical Engineering, Bucknell University, Lewisburg, PA 17837, USA (e-mail: cbeal@bucknell.edu)

Daniel Fescenmyer is a undergraduate student in Mechanical Engineering at The Pennsylvania State University, University Park, PA 16802, USA (e-mail: dzf5248@psu.edu).

Sean Brennan is with the Department of Mechanical Engineering, The   Pennsylvania State University, University Park, PA 16802, USA (e-mail: sbrennan@psu.edu). IEEE Member.